ENGN2020 – HOMEWORK8

### Problem 1

### Part (a)

0=-vdCdz+ka(Csat-C)

The initial condition is that at z = 0, where is exactly the outlet of O2, C = 0. That`s Cz=0=0.

Let:

θ≡ C-Csat-Csat, then dθdz=-1CsatdCdz

The original equation can be written as:

vdθdz= -kaθ

Let:

l≡ zvka=kazv, then dzdl=vka,

The equation above can be written as:

dθdl= -θ

Where:

θ= Csat-CCsat

l= zvka=kazv

The initial condition becomes:

θl=0=1

### Part (b)

Rdqdt+1Cq= V

The equation can be written as:

Rdqdt=1C(VC-q)

Let:

θ≡q-VC-VC, then dθdt=-1VCdqdt,

The equation can be written as:

RCdθdt=-θ

Let:

τ≡ tRC, then dtdτ=RC,

The equation can be written as:

dθdτ=-θ

Where:

θ=q-VC-VC

τ= tRC

The initial condition becomes:

θτ=0=1

### Problem 2

mCPdTdt= -hAT-T∞+a(1+sinbt)

Where:

m=1kg

a=200W

b=0.1 s-1

A=0.02 m2

T0=25 °C

The function can be rewritten as:

dTdt= -hAmCPT-T∞+amCP(1+sinbt)

In python, the function can be defined as:

import math

from scipy.integrate import odeint

'''

* @name: f_fit

* @description: the function to be fitted by the experimental points

* @param t: time, in form of array

* @param cp: heat capacity

* @param h: surface heat-transfer coefficient

* @return: the corresponding temperature of given t

'''

def f_fit(t,cp,h):

#copy the input parameter of cp and h

alpha = cp

beta = h

#the pre-defined parameter

m = 1

a = 200

b = 0.1

A = 0.02

T_inf = 25

#define the function to be solved by numerical method in lambda format

f = lambda y,t: -beta*A*(y-T_inf)/m/alpha + a * (1+ math.sin(b*t))/m/alpha

#set the initial guess by T0

y0 = 25

#solve for temperature of given time

results = odeint(f, y0, t)

#resize the temperature as 1*141

results.resize((141))

return results

The experiment data can be loaded by following code:

import numpy as np

import matplotlib.pyplot as plt

from scipy.optimize import curve_fit

#load the experimental data

data = np.load('thermal-block.npz')

#get time

t = data['times']

#get temperature

realTemperature = data['temperatures']

#resize the temperature as the same size with time

realTemperature.resize((141))

Use scipy.optimize.curve_fit to fit for the experimental results to calculate cp and h. After calculation,

CP= 1395.71665065J/kg/K

h= 871.58696023W/m2/K

#curve fit to calculate cp and h

popt, pcov = curve_fit(f_fit, t, realTemperature)

#calculate the temperature based on fitting result

fitTemperature = f_fit(t,popt[0],  popt[1])

fig, axs = plt.subplots(1, 1)

axs.plot(t,fitTemperature,'r')

axs.plot(t,realTemperature,'bx')

axs.set_xlabel('time')

axs.set_ylabel('CA')

plt.show()

The plot containing the best-fit model solution and the experimental points is shown in Fig.1.

Fig 1. The plot containing the best-fit model solution and the experimental points

### Problem 3

### Part (a)

According to the problem:

ρCp∂T∂t=k∂2T∂x2⋯⋯⋯⋯⋯①

At x = 0:								-kA∂T∂x=-hAT-T∞⋯⋯⋯②

At x = L:								-kA∂T∂x=0⋯⋯⋯⋯⋯⋯⋯⋯③

Let θ≡ T-T∞T0-T∞,then ∂θ∂x= 1T0-T∞∂T∂x, ∂θ∂t= 1T0-T∞∂T∂t, ∂2θ∂x2= 1(T0-T∞)2∂2T∂x2

Then, equation 3 becomes:

-kAT0-T∞dθdx=0

Let z≡ xL, then ∂θ∂x=1L∂θ∂z, ∂2θ∂x2= 1L2∂2θ∂z2

Then, equation 3 becomes:

∂θ∂z=0,  at z=1

Then equation 2 becomes:

∂θ∂z=hLkθ,  at z=0

Let η≡ hLk, then equation 2 becomes:

∂θ∂z=ηθ,  at z=0

From above ∂2θ∂x2= 1(T0-T∞)2∂2T∂x2 and ∂2θ∂x2= 1L2∂2θ∂z2, then:

∂2T∂x2= (T0-T∞)2L2∂2θ∂z2

Then equation 1 becomes:

ρCpT0-T∞∂θ∂t=kT0-T∞2L2∂2θ∂z2

Then:

L2ρCpkT0-T∞∂θ∂t=∂2θ∂z2

Let τ= t L2ρCpkT0-T∞, then:

∂θ∂τ=∂2θ∂z2

In conclusion:

∂θ∂τ=∂2θ∂z2∂θ∂z=ηθ, at z=0∂θ∂z=0, at z=1

Where:

θ= T-T∞T0-T∞z= xL η= hLkτ= kT0-T∞ L2ρCpt

### Part (b)

Fig 2. The finite element model

To discretizing these equations:

∂2θ∂z2=θi+1-2θi+θi-1∆z2

∂θ∂z= θi+1-θi-12∆z

The first boundary condition becomes:

At z=0,  ∂θ∂z|i=0=θ1-θ-12∆z=ηθ0, then θ-1= θ1-2∆zηθ0

Therefore:

∂θ∂τ|i=0=∂2θ∂z2|i=0=θ1-2θ0+θ-1∆z2=2θ1-2θ0-2∆zηθ0∆z2

The second boundary condition becomes:

At z=1,  ∂θ∂z|i=n-1=θn-θn-22∆z=0, then θn= θn-2

Therefore:

∂θ∂τ|i=n-1=∂2θ∂z2|i=n-1=θn-2θn-1+θn-2∆z2=2θn-2-2θn-1∆z2

The governing function becomes:

∂θ∂τ=θi+1-2θi+θi-1∆z2
The code for class Derivatives is shown as below:

class Derivatives:

'''

* @name: __init__

* @description: the constructor of class

* @param n: parameter of number of elements

* @param eta: parameter of eta

'''

def __init__(self,n,eta):

self.n = n

self.step = 1/n

self.eta = eta

'''

* @name: __call__

* @description: the call function

* @param y: current parameter for 'theta'

* @param t: input value of 'tao'

* @return: dy, the value of dydt

'''

def __call__(self, y, t):

dy = np.zeros((self.n))

#the boundary condition at z=0

dy[0] = (2*y[1]-2*y[0]-2*self.step*﻿self.eta*y[0])/self.step/self.step

#the governing function

for i in range(1,self.n-1):

dy[i] = (y[i+1]- 2*y[i]+y[i-1])/self.step/self.step

#the boundary condition at z=n-1

dy[self.n-1] = (2*y[self.n-2]-2*y[self.n-1])/self.step/self.step

#return dy

return dy

### Part (c)I

In order to show the effect of η, the plots of θ versus t at different positions, namely z = 0, z = 0.5, z = 1 are shown in Fig.3.

（a）η = 0.1										(b) η = 0

(c）η = 1										(d) η = 10

Fig 3. plots of θ versus t at different positions, namely z = 0, z = 0.5, z = 1

From Fig.3, it can be found that, when η = 0, the left end is also perfectly insulated, there will be no heat exchange between block and environment, the block keeps its original temperature.

When η ≠ 0, as η increases, the system takes less time to reach the stable state. So η has effect on the “speed” of transferring heat.

To solve this problem numerically, the following code was used. In this piece of code, ﻿scipy.integrate.odeint is used.

'''

* @name: solve

* @description: the function to get solve the finite element problem

* @param n: the number of finite elements in the block

* @param eta: parameter of transferring heat

'''

def solve(n,eta):

#define a object based on input n and eta

dydt = Derivatives(n, eta)

#get the initial guess of y0

y0 = np.ones((n))

#get t

t = np.linspace(0,10,1000)

#solve for y

y = odeint(dydt, y0, t)

#show y versus t at different positions, z = 0, z=0.5, z=1

fig, axs = plt.subplots(3, 1)

axs[0].plot(t,y[:,0],'r')

axs[0].set_xlabel('time')

axs[0].set_ylabel('theta at z = 0')

axs[1].plot(t,y[:,n//2],'g')

axs[1].set_xlabel('time')

axs[1].set_ylabel('theta at z = 0.5')

axs[2].plot(t,y[:,n-1],'b')

axs[2].set_xlabel('time')

axs[2].set_ylabel('theta at z = 1')

plt.show()